Protocol for Elderly Care

Smart Watch

V1.0

Communication protocol

Copyright statement

all rights reserved.

Unauthorized copy or distribution of this document, in whole or in part will bear all legal responsibilities.

Revised record:

| 2023/2/1 | Engineer kaka | original version |
| --- | --- | --- |

Content

1. Communication agreement	3

二：Commands sent from end user (device server)	5

1.	AP00-Login package(responds:BP00)	5

2.	AP01 Locating package, GPS+LBS+Status+Base +WIFI combining package (responds:BP01)	6

3.	AP02 Multiple bases locating package (responds:BP02)	7

4.	AP03 Heartbeat package(responds:BP03)	8

5.	AP07 Upload audio message（responds:BP07）	8

6.	AP10 Alarm and Return address Package(responds:BP10)	9

7  AP49 Upload heart rate（responds：BP49）	11

8	APHT  Upload heart rate and BP（responds：BPHT）	11

三：Commands sent from server（server device）	12

1.	BP12 Set sos numbers(three)（respond：AP12）	12

2.	BP14 Set white list(Phone book) numbers(ten)（respond：AP14）	12

3.	BP16 Real-time locating command（respond：AP16）	13

4.	BP17 Factory reset（respond：AP17）	13

5.	BP28 Send audio message to device（responds:AP28）	14

6.	BP33 Working Mode（respond：AP33）	15

7.	BP84 Switch for White list（respond：AP84）	15

8.	BP85 Set an alarm/reminder（respond：AP85）	16

9.	BPJZ  Blood pressure calibration（respond：APJZ）	16

10.	BPXL Test heart rate（respond：APXL）	17

11.	BPXY  Test blood pressure（respond：APXY）	17

12.	BPXT  Test temperature（respond：APXT）	18

13.	BP87  set the interval of auto Test temperature（respond：AP87）	19

1. Communication agreement

1.1 Introduction

This  document  defines  the  GPS  locator  positioning  service  platform  for  the  application  layer interface  protocols.The  relevant  interface  protocols Only  apply  to  the  interaction  between  the  platform and positioning terminal.

1.2 Network connection way

Long connection is through 2G GSM/4G LTE using TCP connection. The relationship between watch and server and app is shown in the figure below. Watch send data(login data and location data and health data) to server by mobile network; Server can reply watch and send command to watch. App and web can get data from server and show to user.

1.3 Terms and definitions

| Terms abbreviations | Meaning |
| --- | --- |
| CMPP | China Mobile Peer to Peer |
| GPS | Global Positioning System |
| GSM | Global System for Mobile Communication |
| GPRS | General Packet Radio Service |
| TCP | Transport Control Protocol |
| LBS | Location Based Services |
| IMEI | International Mobile Equipment Identity |
| MCC | Mobile Country Code |
| MNC | Mobile Network Code |
| LAC | Location Area Code |
| CI | Cell ID |
| RSSI | Received Signal Strength Indicator |
| UDP | User Datagram Protocol |
| SOS | Save Our Ship/Save Our Souls |
| CRC | Cyclic Redundancy Check |
| NITZ | Network Identity and Time Zone, |
| GIS | Geographic Information System |

1.4 Basic rules

(1)  GPRS connection is established successfully and send the first landing a packet to the server, 5 seconds  received  server  response  packet  that  connection  is  normal,  start  sending  location  information (GPS, LBS packets), 3 minutes will send packets, regular communication is normal.

(2)   When the  GPRS connection is not  successful, the terminal  can't send packets.When the  GPRS connection failed three times  start timing restart  function after the  final end, time  for 20 minutes.In  20 minutes  if  the  terminal  and  server  set up  even  after  success,  and received  the  terminal  server to  send packets  to respond packets  of the  login, the restart  function off timer,  the  terminal  does not  restart,  or terminal automatic restart after 20 minutes

(3)  After receiving terminal to  send the login information to the  server terminal returns a response packet, if the end to send the login information package or state packet after more than five seconds didn't receive  the  server  returns  package,   argues  that  the  current  abnormal    connection   is  established,  and dynamic GPS positioning data transfer function, disconnect the current GPRS, login and send packets to establish new GPRS connection.

(4)  Connection is judged  as abnormal, repeat 3 times after establishing a connection to send packets of the login or condition of server response packet packets, terminal launch timing reset function, timing time of  10 minutes, in  10 minutes if the end client and the server successfully establish a connection and receive the server response packet restart function off timer, terminal not heavy qi, or terminal automatic restart after 10 minutes.

(5)  After normal  connected, terminal  after the  GPS information change, regularly  send GPS, LBS merger packets sent to the server, the server can be sent via command to set the default protocol.

(6)  Server for the terminal without registering IMEI number, please reply the login request response and heartbeat packets response, do not disconnect even pick up directly.(if directly disconnected or does not reply, will lead to terminal constantly rewiring, GPRS traffic consumption serious)

(7)  All data packages in this protocol refer to formats below:

Head: IW

Protocol number: Uplink(deviceserver)AP[two digits],Downlink(serverdevice)BP[two digits],same digits show data packages respond.

Parameter: content of data package

Terminator:#

All information related with Chinese use UNICODE, such as address.

All punctuation marks in the data packages (except the punctuation in issued address) are English half Angle.

二：Commands sent from end user (device server)

- AP00-Login package(responds:BP00)

| Example: |  |
| --- | --- |
| IWAP00353456789012345# |  |
| Notice: |  |
| IW: Identifier AP00:Command word 353456789012345: IMEI number of devices, default as 15 digits #:Terminator |  |
| platform responds: | platform replies: IWBP00,20150101125223,8# 20150101125223 is server time service, the format is year month date hour minute second, it’s UTC0 time. 8 is the present time zone of server |
| Notice: | New login package will be sent every time the device connects with server. |

- AP01 Locating package, GPS+LBS+Status+Base +WIFI combining package (responds:BP01)

| Example: |  |
| --- | --- |
| IWAP01080524A2232.9806N11404.9355E000.1061830323.8706000908000102¦2@bt0¦23-46-89-ab-cd-ef¦23&bt2¦23-46-89-ab-cd-f1¦38,460,0,9520,3671,Home¦74-DE-2B-44-88-8C¦97& Home1¦74-DE-2B-44-88-8C¦97&Home2¦74-DE-2B-44-88-8C¦97& Home3¦74-DE-2B-44-88-8C¦97# |  |
| Notice: |  |
| IW Identifier AP01: Command word 080524: 24th May 2008 A:“A”shows valid data,”V”shows invalid data and will get LBS data 2232.9806N11404.9355E000.1: 22 degrees north latitude 32.9806 points，114 degrees east longitude 04.9355 points，speed is 000.1 km/h，it’s default as 0 if the latitude and longitude are invalid, such as 0000.0000N00000.0000E 061830: GMT 06:18:30 323.87:direction angle is 323.87° 06000908000102: 060 is GSM signal,009 is the number of satellites,080 is battery level, 0 is remaining space,01 is fortification state,02 is working mode, (it shows none setting if fortification and working mode are 00) ¦2@bt0¦23-46-89-ab-cd-ef¦23&bt2¦23-46-89-ab-cd-f1¦38 Beacon mac address 460,0,9520,3671 : LBS is base data MCC is country code,460 means China,0:MNC,0 is moving,9520:LAC,decimal,3671,CID, decimal If state in GPS package is V or latitude and longitude are “ 0000.0000N00000.0000E”,it will get LBS data. Home¦74-DE-2B-44-88-8C¦97 : one set of WIFI information，Home is SSID， 74-DE-2B-44-88-8C is MAC address，97 is signal strength，variables are separated by“¦”,wifi information can be multiple sets and are separated by “&”. |  |
| platform responds: | platform replies: IWBP01# |
| Notice: |  |

- AP02 Multiple bases locating package (responds:BP02)

| Example： |  |
| --- | --- |
| IWAP02,zh_cn¦2@bt0¦23-46-89-ab-cd-ef¦23&bt2¦23-46-89-ab-cd-f1¦38,0,7,460,0,9520¦3671¦13,9520¦3672¦12,9520¦3673¦11,9520¦3674¦10,9520¦3675¦9,9520¦3676¦8,9520¦3677¦7,4,1¦D8-24-BD-79-FA-1F¦59&2¦3C-46-D8-6D-CE-01¦81&3¦0C-4C-39-1A-7C-65¦69&4¦70-A8-E3-5D-D7-C0¦65# |  |
| Notice: |  |
| IW Identifier AP02: Command word zh_cn: language notice ¦2@bt0¦23-46-89-ab-cd-ef¦23&bt2¦23-46-89-ab-cd-f1¦38 Beacon mac address 0:reply flag，the server doesn’t reply address when it’s 0,and replies address when it’s 1. 7: 7 sets of bases 460: MCC is country code 0: MNC operator code 9520¦3671¦10: LAC¦CID¦dbm shows a set of base information and the numbers should correspond to the number of bases. 10：dbm is signal strength, multiple bases signal strength is suggested to be arranged from high to low then downloaded, signal strength are using absolute values: 150 – abs(dbm) 4: sets of wifi 1¦D8-24-BD-79-FA-1F¦59: ssid¦mac address¦ signal strength，signal strength are using absolute values:150 – abs(signal strength) |  |
| platform responds: | platform replies: IWBP02# If the reply flag is 1 need responds: IWBP02F16D3357025E5753715C3A535753776D275953903100300037003900F753# UNICODE encoded reply message from server which contains address（深圳市南山区南海大道1079号） |
| Notice: |  |

- AP03 Heartbeat package(responds:BP03)

| Example： |  |
| --- | --- |
| IWAP03,06000908000102,5555,30# |  |
| Notice: |  |
| IW: Identifier AP03: Command word Heartbeat message , Device can use this message to keep long connection with server 06000908000102: 060 is GSM signal , not use 009 is the number of satellites 080 is battery level 0 is remaining space 01 is fortification state, Here shows by two hexadecimal,00 if no value, the length should be two digits. There’re 8 digits in total and each digit is defined as below (unlisted one is unused states,1 shows effective and 0 shows invalid) Bit0 fortification state<the device is invalid> Bit1 :1 shows the night-light open,0 shows night-light off 02 is working mode, (it shows none setting if fortification and working mode are 00) 5555: counting steps. Recount from 0 after reaching 9999 steps (9999 is for an example, here is 2 bytes int and overflow to zero) 30: Rolls frequency |  |
| platform responds: | platform replies: IWBP03# |
| Notice: |  |

- AP07 Upload audio message（responds:BP07）

| Example： |  |
| --- | --- |
| IWAP07,20140818064408,6,1,1024,XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX# |  |
| Notice: |  |
| IW：Identifier AP07 : Command word 20140818064408 :yyyyMMddHHmmss ，device record time 6：the total number of audio data packet 1 ：sequence of the audio data packet 1024 :the length of audio data XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX：audio data as bytes, each packet can not beyond 1024 byte，the last audio data packet maybe less than 1024 byte, if less than 1024 byte then send length as actual size The rule for audio upload： 1: if device not received the responds from server, it will repeat upload the same audio data packet 2: if the server responds that received succesfully, device will upload the next audio data packet if current sequence not reach the total packet number 3: if the server responds that received failed, it will repeat upload the same audio data packet |  |
| platform responds: | IWBP07,20140818064408,6,1,1# 20140818064408：device record time 6：the total number of audio data packet 1: current sequence of the audio data packet the sever received 1：the server responds that received succesfully，0:the server responds that received failed |
| Notice: | Now only support amr audio between device and server |

- AP10 Alarm and Return address Package(responds:BP10)

| Example： |  |
| --- | --- |
| IWAP10080524A2232.9806N11404.9355E000.1061830323.8706000908000502,460,0,9520,3671,00,zh-cn¦2@bt0¦23-46-89-ab-cd-ef¦23&bt2¦23-46-89-ab-cd-f1¦38,00,HOME¦74-DE-2B-44-88-8C¦97&HOME1¦74-DE-2B-44-88-8C¦97&HOME2¦74-DE-2B-44-88-8C¦97&HOME3¦74-DE-2B-44-88-8C¦97# |  |
| Notice: |  |
| IW : Identifier AP10: Command word 080524: 24th May 2008 A: “A”shows valid data,”V”shows invalid data and will get LBS data 2232.9806N11404.9355E000.1: 22 degrees north latitude 32.9806 points，114 degrees east longitude 04.9355 points，speed is 000.1 km/h，it’s default as 0 if the latitude and longitude are invalid, such as 0000.0000N00000.0000E 061830: GMT 06:18:30 323.87: direction angle is 323.87° 06000908000102: 060 is GSM signal,009 is the number of satellites,080 is battery level, 0 is remaining space,01 is fortification state,02 is working mode, (it shows none setting if fortification and working mode are 00) 460,0,9520,3671 : LBS is base data MCC is country code,460 means China,0:MNC,0 is moving,9520:LAC,decimal,3671,CID, decimal 00 is alarm state,00 is no alarm (01：SOS,02：low battery,06：fall down alarm,04:wearing notice) zh-cn: device language ¦2@bt0¦23-46-89-ab-cd-ef¦23&bt2¦23-46-89-ab-cd-f1¦38 Beacon mac address 00:The first one 0:whether the reply address is needed,0:no reply,1 reply. The second 0:whether mobile hyperlink is contained in address information,0 not contained,1 contained Home¦74-DE-2B-44-88-8C¦97 : one set of WIFI information，Home is SSID， 74-DE-2B-44-88-8C is MAC address，97 is signal strength，variables are separated by“¦”,wifi information can be multiple sets and are separated by “&”. |  |
| platform responds: | platform responds BP10,if return address is not needed, there should be space in return package. IWBP106df157335e0253575c71533a53576d7759279053003100300037003953f7002000200068007400740070003a002f002f007700770077002e006700700073002e0063006f006d002f006d00610070002e0061007300700078003f006c00610074003d00320033002e0031003200330026006c006e0067003d003100310033002e003100320033# UNICODE encoded reply message from server which contains address, above example shows: 深圳市南山区南海大道1079号http://www.gps.com/map.aspx?lat=23.123&lng=113.123 The language and content automatically judge according to langiage inAP10 package, whether to reply hyperlink according to states of AP10. |
| Notice: |  |

7  AP49 Upload heart rate（responds：BP49）

| Example： |  |
| --- | --- |
| IWAP49,68# |  |
| Notice: |  |
| IW：Identifier AP49 : Command word 68 : Heart rate |  |
| platform responds: | IWBP49# |
| Notice: |  |

- APHT  Upload heart rate and BP（responds：BPHT）

| Example： |  |
| --- | --- |
| IWAPHT,60,130,85# |  |
| Notice: |  |
| IW：Identifier APHT : Command word 60 : Heart rate 130: systolic pressure 85 : diastolic pressure |  |
| platform responds: | IWBPHT# |
| Notice: |  |

- APHP Upload heart rate、BP、SPO2、blood sugar（responds：BPHP）

| Example： |  |
| --- | --- |
| IWAPHP,60,130,85,95,90,,,,,,,,# |  |
| Notice: |  |
| IW：Identifier APHT : Command word 60 : heart rate 130:blood pressure[SBP] 85 : blood pressure[DBP] 95：SPO2 90：blood sugar |  |
| platform responds:： | IWBPHP# |
| Notice： | Each value is not necessary.if there is no value, the packet leave it blank |

- AP50 Upload body temperature （responds：BP50）

| Example： |  |
| --- | --- |
| IWAP50,36.7,90# |  |
| Notice: |  |
| IW：Identifier AP50 : Command word 36.7 : body temperature 90：device battery level |  |
| platform responds:： | IWBP50# |
| Notice： | Each value is not necessary.if there is no value, the packet leave it blank |

- ECG upload（upload：APHD，reply：BPHD）Optional function

| Example： |  |
| --- | --- |
| IWAPHD,20200707111800,6,1,1.3,20.2,1,1000,1024,XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX# |  |
| Notice: |  |
| IW：Identifier APHD: Command word 20200707111800: date and time; 2020Y 7m 7d 11h8m00s 6: 6 data packets 1：now it is the first data packet 1.3: gain 20.2：0 point voltage 1：Lead number 1000： sampling rate 1024：Data length，max is 1024 XXXXXXXXXXXXXX: Ecg data |  |
| Server reply： | IWBPHD,0, 98, 59, 156, 296, 369,7aa660275fc35f8b4e0d9f50¦623f98a4 # 0：AI analysis OK 1：AI analysis NG 98: average heart rater 59: Qrs 156: PR 296: Q-t 369: QTc 7aa660275fc35f8b4e0d9f50¦623f98a4：atrial fibrillation |
| Notice: |  |

三：Commands sent from server（server device）

- BP12 Set sos numbers(three)（respond：AP12）

| Example： |  |
| --- | --- |
| IWBP12,353456789012345,080835,135XXXXXXXX,135XXXXXXXX,135XXXXXXXX# |  |
| Notice: |  |
| IW：Identifier BP12 : Command word 353456789012345:IMEI number and the unique ID of the device 080835 : journal no. 135XXXXXXXX,135XXXXXXXX,135XXXXXXXX : SOS numbers,max set 3 SOS, all the phone no. should be set one by one , If one of the SOS not be set, the corresponding position leave space. |  |
| device responds： | IWAP12,080835,135XXXXXXXX,135XXXXXXXX,135XXXXXXXX# 080835: The device responds with response journal no. |
| Notice: |  |

- BP14 Set white list(Phone book) numbers(ten)（respond：AP14）

| Example： |  |
| --- | --- |
| IWBP14,353456789012345,080835,D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx# |  |
| Notice: |  |
| IW：Identifier BP14 : Command word 353456789012345: IMEI number and the unique ID of the device 080835 : journal no. D3590D54¦135xxxxxxxxxx : A set of contacts and use ¦ to separate name and phone number, names encode UNICODE and different sets are separated by a comma, If a phone number is not set, the corresponding position leave space. |  |
| device responds： | IWAP14,080835, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx, D3590D54¦135xxxxxxxxxx# 080835: The device responds with response journal no. |
| Notice: |  |

- BP16 Real-time locating command（respond：AP16）

| Example： |  |
| --- | --- |
| IWBP16,353456789012345,080835# |  |
| Notice: |  |
| IW：Identifier BP16 : Command word 353456789012345: IMEI number and the unique ID of the device 080835 : journal no. |  |
| device responds： | IWAP16,080835# 080835: journal no. device will Async send AP01 location data after responds. |
| Notice: |  |

- BP17 Factory reset（respond：AP17）

| Example： |  |
| --- | --- |
| IWBP17,353456789012345,080835# |  |
| Notice: |  |
| IW：Identifier BP17 : Command word 353456789012345: IMEI number and the unique ID of the device 080835 : journal no. |  |
| device responds： | IWAP17,080835# 080835: journal no. |
| Notice: |  |

- BP28 Send audio message to device（responds:AP28）

| Example： |  |
| --- | --- |
| IWBP28, D3590D54,XXXX,6,1,1024,XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX# |  |
| Notice: |  |
| IW：Identifier BP28 : Command word D3590D54: UNICODE，the sender name XXXX: additional data 6：the total number of audio data packet 1 ：sequence of audio data packet 1024 :the length of data packet XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX：audio data, each audio can not beyond 1024 byte，the last audio data packet maybe less than 1024 byte, if less than 1024 byte then send length as actual size The rule for audio upload： 1: if server don’t received the responds from device, it will repeat send the same audio data packet 2: if the device responds that received succesfully, it will send the next audio data packet 3: if the device responds that received failed, it will repeat send the same audio data packet |  |
| device responds： | IWAP28,D3590D54,XXXX,6,1,1# D3590D54:UNICOD，sender name XXXX: additional data,return the same content received from the server 6：the total number of audio data packet 1：sequence of the audio data packet the device received 1：device have received the audio data，0:device don’t have received the audio data |
| Notice: | Now only support amr audio between device and server |

- BP33 Working Mode（respond：AP33）

| Example： |  |
| --- | --- |
| IWBP33,353456789012345,080835,1# |  |
| Notice: |  |
| IW：Identifier BP33 : Command word 353456789012345: IMEI number and the unique ID of the device 080835: journal no. 1：working mode of the device， 1：normal mode，every 15minute a position report with WIFI and lbs 2：Power-saving mode，every 60 minute a position report with WIFI and lbs 3：emergency mode 1 minute a position report with GPS and WIFI and lbs Mode 8 : IWBP33,353456789012345,080835,8,30,1# This is a special work mode, you can set any interval (at least 30 seconds) and you can ON/OFF GPS 8: mode 8 30 seconds is the interval of the location report 1: is mean GPS is ON and if it is 0 mean GPS off |  |
| device responds： | IWAP33,080835,1# 080835: journal no. |
| Notice: |  |

- BP40：Send text to watch（respond：AP40）

| Example： |  |
| --- | --- |
| IWBP40,353456789012345,080835,00610072006500200079006f00750020006f006b003f# |  |
| Notice: |  |
| IW：Identifier BP40 : Command word 353456789012345: IMEI 080835: journal no. 00610072006500200079006f00750020006f006b003f UNICODE decode , mean： ‘are you ok?’ |  |
| device responds： | IWAP40,080835# |

8  BP76:  Fall alert switch（respond：AP76）

| Example： |  |
| --- | --- |
| IWBP76,353456789012345,080835,1# |  |
| Notice: |  |
| IW：Identifier BP76 : Command word 353456789012345: IMEI 080835: journal no. 1/0: switch on/switch off |  |
| device responds： | IWAP76,080835,1# |
| Notice: |  |

9  BP77: Set fall alarm sensitivity（respond：AP77）

| Example： |  |
| --- | --- |
| IWBP77,353456789012345,080835,1# |  |
| Notice: |  |
| IW：Identifier BP77 : Command word 353456789012345: IMEI 080835: journal no. 1/2/3: sensitivity， 3 represents the highest sensitivity |  |
| device responds： | IWAP77,080835# |
| Notice: |  |

- BP84 Switch for White list（respond：AP84）

| Example： |  |
| --- | --- |
| IWBP84,353456789012345,080835,1# |  |
| Notice: |  |
| IW：Identifier BP84 : Command word 353456789012345: IMEI number and the unique ID of the device 080835 : journal no. 1: white list on 0: white list off |  |
| device responds： | IWAP84,080835,1# 080835: journal no. 1：device have run the command , 0：device don’t have run the command |
| Notice: |  |

- BP85 Set an alarm/reminder（respond：AP85）

| Example： |  |
| --- | --- |
| IWBP85,353456789012345,080835,1,3, 0900,135,1,1@0900 ,135,1,2@0900 ,135,1,3# |  |
| Notice: |  |
| IW：Identifier BP85 : Command word 353456789012345: IMEI number and the unique ID of the device 080835 : journal no. 1: master switch for all alarm，1 is on，0 is off 3: total number of all alarm 0900 ,135,1,1 : a reminder，0900 mean reminder time 09:00, 135 mean Monday,Wednesday,Friday ，24 hour system 1 switch for this alarm 1 is on 0 is off 1 Take the medicine reminder 2 Drink water reminder 3 A sedentary reminder Each alarm Separated by ”@” |  |
| device responds： | IWAP85, 080835,1,3, 0900,135,1,1@0900 ,135,1,2@0900 ,135,1,3# 080835:The device responds with response journal no. |
| Notice: |  |

- BP86 Set the interval of all hearth data auto testing（respond：AP86）

| Example： |  |
| --- | --- |
| IWBP86,353456789012345,080835,1,720# |  |
| Notice: |  |
| IW：Identifier BP86 : Command word 353456789012345: IMEI number and the unique ID of the device 080835 : journal no. 0: close measurement; 1: turn on auto measurement; 720 is 720minutes |  |
| device responds： | IWAP86, 080835# 080835:The device responds with response journal no. |
| Notice: | Server send this command to watch, watch will test all health data and send to server according the interval. |

- BPXL Test heart rate（respond：APXL）

| Example： |  |
| --- | --- |
| IWBPXL,353456789012345,080835# |  |
| Notice: |  |
| IW：Identifier BPXL : Command word 353456789012345: IMEI number and the unique ID of the device 080835 : journal no. |  |
| device responds： | IWAPXL,080835# 080835:The device responds with response journal no. |
| Notice: |  |

- BPXY  Test blood pressure（respond：APXY）

| Example： |  |
| --- | --- |
| IWBPXY,353456789012345,080835# |  |
| Notice: |  |
| IW：Identifier BPXY : Command word 353456789012345: IMEI number and the unique ID of the device 080835 : journal no. |  |
| device responds： | IWAPXY,080835# 080835:The device responds with response journal no. |
| Notice: |  |

- BPXT  Test temperature（respond：APXT）

| Example： |  |
| --- | --- |
| IWBPXT,353456789012345,080835# |  |
| Notice: |  |
| IW：Identifier BPXT : Command word 353456789012345: IMEI number and the unique ID of the device 080835 : journal no. |  |
| device responds： | IWAPXT,080835# 080835:The device responds with response journal no. |
| Notice: | This command used to test once temperature |

- BPXZ  Test SPO2 once from server（respond：APXZ）

| Example： |  |
| --- | --- |
| IWBPXZ,353456789012345,080835# |  |
| Notice: |  |
| IW：Identifier BPXZ : Command word 353456789012345: IMEI number and the unique ID of the device 080835 : journal no. |  |
| device responds： | IWAPXZ,080835# 080835:The device responds with response journal no. |
| Notice: | This command used to test once spo2 from server |

三：Some questions and answers

SMS commands list：

| Set server’s ip and port |  |
| --- | --- |
| Format | #host#=Domain name,port# or #IP#=ip address,port# |
| Example | #host#=api.gswatch.com,5088# or #ip#=198.11.183.28,5088# |
| Reply sms | Host set ok or IP set ok |
| Add a new apn |  |
| Format | #apn#=MCC,MNC,APNName,AccountName,UserName, PW,Authen Type# |
| Example | #apn#=460,02,cmnet,CMNET,null,null,PAP# |
| Reply sms | Apn set ok |
| Set the sos number to watch |  |
| #sos1#=008613711111111# | Set the first sos number is 008613711111111 |
| #sos2#=008613722222222# | Set the second sos number is 008613722222222 |
| #sos3#=008613733333333# | Set the third sos number is 008613733333333 |
| Check imei and firmware version |  |
| Format | #status# |
| Example | #status# |
| Reply sms | IMEI:357653050444319; VER:H10_V1.0; |
| Add a new phonebook to watch |  |
| Format | #phbadd# |
| Example | #phbadd#=13812222222,name# |
| Reply sms | Add phb: ok |
| Set the interval of the location and switch on/off GPS |  |
| Format | #mode#=8,interval,GPS switch# |
| #mode#=8,181,1# | Mean:GPS is on, and the interval of location is 181 seconds |
| #mode#=8,181,0# | Mean:GPS is off,and the interval of location is 181 seconds |
| Get location from watch: send sms command to watch to get a last location |  |
| Format | #location# |
| Example | #location# |
| Reply sms | GPS location: 113.996620,22.641510 |
| Power off the watch |  |
| Format | #shutdown# |
| Example | #shutdown# |
| Reply sms | ok |
| Restart the watch |  |
| Format | #reboot# |
| Example | #reboot# |
| Reply sms | ok |
| Make the watch back to factory setting |  |
| Format | #factoryreset# |
| Example | #factoryreset# |
| Reply sms | No reply |
| ON/Off get time from server |  |
| Format | #timeauto:on# #timeauto:off# |
| Example |  |
| Reply sms | timeauto:on OK! timeauto:off OK! |
| Set time for watch: if you want set watch time by sms, you should off getting time from server first |  |
| Format | #timedate: |
| Example | #timedate:2019.06.18,19:24 |
| Reply sms | #timedate:2019.06.18,19:24 ok |
| Set the interval of heartbeat (AP03) |  |
| Format | #hb#=time# |
| Example | #hb#=5# |
| Reply sms | Set ok(mean set the interval of AP03 is 5 minutes) |